BF100 Carburetor adjustment

Idle speed:

1. Start the engine and run at idle until normal operating temperature is obtained.

2. Turn the throttle stop screw in or out as necessary until the specified idle speed is obtained.

Pilot screw:

If the pilot screw setting needs adjustment.

Normal setting 2,5 turn out

1. Turn the pilot screw in or out until the highest idle speed RPM is reached

2. Readjust to the specified idle speed using the throttle stop screw
